C/C++ Interface description of SerialSource methods implemented in portiserial.dll version 2.0.72.0

HRESULT put_ComPort([in] BSTR newVal);

| purpose: | Sets the name of the comport used and opens this |
| --- | --- |
| in: | newVal = name of comport to open, should be in format “com*” or “*” with * the number of the comport., if newVal represents an incorrect comport the function will return an error |
| out: |  |
| return: | S_FALSE if one of the following errors occurs: -Portname is incorrect -The port can’t be opened -Device info can’t be retrieved (data can’t be send or is not acknowledged) S_OK if no error |
| process: | -Initializes a serial port for reading and writing to the device -Starts a 2nd thread for retrieving measurement data -Retrieves device info (frontend info, id data, channel size/type) and sets the according variables in pFrontend and pDataStream |
| assumptions: | Device is connected and turned on and serial port is working |

HRESULT get_ComPort([out, retval] BSTR *pVal);

out: 		pVal contains a string with the comport name

remark: 	if no device at this comport pVal will be the string:"no device opened”

STDMETHOD(InitilizeSerialPortAndFrontend)([in] BSTR ComPortName, [in] long Baudrate, [out] long *pErrorResult);

Process:	same as put_comport, return error in pErrorResult, -1 if failure, BaudRate is not used

STDMETHOD( get_Caption )( [out,retval]BSTR* pstrCaption);

Process:	Reads the name of the device

When opening device set from id_data

STDMETHOD( put_Caption )( [in]BSTR strCaption );

Process:	sets the name of the device in the software (not in the device)

HRESULT StartAcq( [in, out] float *pFreq, [out] long *pErrorResult);

| purpose: | Start retrieving data from the device |
| --- | --- |
| in: | -pFreq is the sample frequency, the closest frequency supported by the device is set and stored as variable in pFrontend |
| out: | pErrorResult is –1 if error occurred otherwise 0 |
| return: | S_FALSE if the device can’t be set into measurement mode S_OK if no error |
| process: | -Checks if VLDeltaData is supported and retrieves VLDeltaInfo from the device and sets this -Number of used channels, sampleratedivider are send to the device, data mode turned on -A variable in pDataStream is set to store the retrieved data in its buffer |
| assumptions: | -A comport has been opened and the device is turned on |

HRESULT StopAcq([out] long *pErrorResult);

| purpose: | Finishing retrieving data from the device |
| --- | --- |
| in: |  |
| out: | pErrorResult: -1 if error else 0 |
| return: | S_FALSE if device not availlable, else S_OK |
| process: | - Number of used channels, sampleratedivider and are send to the device, data mode turned off |
| assumptions: | -Serial port is opened and device is on |

HRESULT GetSampleRecordAsVariant([out] LONG* pReceivedPeriods,

[out] VARIANT* pVar);

| Purpose: | Gets data from internal buffer |
| --- | --- |
| In: |  |
| Out: | pReceivedPeriods = number of received periods pVar = recorded data |
| Return: | S_FALSE if the device is not availlable and the thread is not running |
| Process: | -the data that is stored in the buffer kept by pDataStream is retrieved and put in the right format. Samples are retrieved untill a buffer of 512 kB is full. |
| Assumptions: | -Serial port is opened and device is availlable -Acquisition is started and buffer is (being) filled with data |

Difference from GetSampleRecordAsVariant is that we can ask for a certain nr of samples, the real number retrieved is written back to pNrOfPeriods

HRESULT GetStationairSampleRecords(	[in,out] LONG* pNrOfPeriods,

[out] VARIANT* pReceiveDataBuffer);

HRESULT KeepSerialPortInUse(	[in] long ReleaseTimeoutTime,

[out] long *pErrorResult);

| purpose: |  |
| --- | --- |
| In: | ReleaseTimeoutTime: timeperiod before this parameter must be updated in milliseconds (0=DisableAutoReleasing) |
| Out: | pErrorResult is –1 if error occurs else 0 |
| Return: | S_FALSE is device is not availlable else S_OK |
| process: | Updates an internal variable before it expires |
| assumptions: | Device is initialized and thread is running |

HRESULT ReleaseSerialPort([out] long *pErrorResult);

| purpose: | Stop thread and free serial port |
| --- | --- |
| In: |  |
| Out: | pErrorResult is 0 in all cases |
| Return: | S_OK is always returned |
| process: | Deletes all devices from the list and stops thread and releases serial port |
| assumptions: |  |

HRESULT TestBlock([in] long BlockTypeSend, [in] long BlockTypeWaitFor,

[in] long NrOfWordsSend,[in,out]long*NrOfWordsReceive,

[in] long *DataToSend, [out] long *ReceiveDataBuffer);

| purpose: | Send any data to the device |
| --- | --- |
| in: | BlockTypeSend: kind of message to send to device BlockTypeWaitFor: expected reply NrOfWordsSend: size of data send to the device NrOfWordsReceive: max number to retrieve |
| out: | ReceiveDataBuffer: data words received from device, if something wrong will be set to –1 NrOfWordsReceive: size of data returned from device |
| return: | S_FALSE if data couldn’t be send or nothing is retrieved or if device not availlable S_OK if message was send and answer was retrieved |
| process: | Sends any message to the device and retrieve the answer |
| assumptions: |  |

The following functions are similar

process: to get the value the device is using

value is not read from the device but from the feInfo variable set in object Frontend

in:

out:	pVal = value to get

return:	S_FALSE if pVal is a NULL pointer or device not opened

S_OK if pVal > 0 after reading from frontend info

HRESULT GetFrontendNrOfChannels([out] long *pNrOfChannels);

HRESULT get_FrontendNrOfChannels(	[out, retval] long *pVal);

HRESULT get_FrontendSerialNumber(	[out, retval] long *pVal);

HRESULT get_FrontendId([out, retval] long *pVal);

HRESULT get_FrontendHardwareVersion([out, retval] long *pVal);

HRESULT get_FrontendSoftwareVersion([out, retval] long *pVal);

HRESULT get_FrontendCommandBufferSize([out, retval] long *pVal);

HRESULT get_FrontendSendBufferSize([out, retval] long *pVal);

For getting and putting the samplerate: the samplerate must still be set by startacquisition

These functions just put the correct value in the software

HRESULT get_SampleRate([out, retval] float *pVal);

HRESULT put_SampleRate([in] float newVal);

HRESULT GetRawData([out] VARIANT* pReceiveDataBuffer, [in, out] LONG * mxSz)

Retrieves the rawdata messages from the buffer into pReceiveDataBuffer with maximum size mxSZ

Out:	mxSZ contains the amount of words really retrieved

HRESULT ResetRawData([in] BOOL Start);

In:	start or stop the rawdata storage

Resets the rawdata storage